PIN-OUT

LINEAR ACTUATOR
MLA-CMA06-25

Connector DB15M(HD)

| Pin# | Signal | Note |
| --- | --- | --- |
| 1 | Motor phase B+ | Current 0.6 Amp pk |
| 2 | Motor phase B- | Current 0.6 Amp pk |
| 3 | Motor phase A+ | Current 0.6 Amp pk |
| 4 | Motor phase A- | Current 0.6 Amp pk |
| 5 | Vcc (sensors) | 5V +/- 5% |
| 6 | - |  |
| 7 | GND (sensors) |  |
| 8 | Out Neg Limit | Iout 20 mA |
| 9 | Out Pos Limit | Iout 20 mA |
| 10 | - |  |
| 11 | - |  |
| 12 | - |  |
| 13 | - |  |
| 14 | - |  |
| 15 | - |  |

Motor: Nanotec ST2018S0604

Sensor: EE-SX1107 amplified with ICNC7SZ32

Positive direction with shaft extended