/*

LinuxCNC COMP function for PUMA lathe toolchanger

Im assuming that this function can never 'stall'.  If it needs to wait for something,

it should instead return control back to LCNC and check the next time its called if enough

time has elapsed.  This requirement lends itself to a 'state machine' based design.

*/

/**************************************************************************************

I/O to LinuxCNC/HAL

**************************************************************************************/

component pumachanger                   "This component controls the Puma Lathe Auto Tool Changer. M6 calls this";

pin in bit toolchange                   "Receives signal from M6 that tool change required";

pin in s32 toolnumber                   "Receives Tx data from M6 (tool number requested) Only allows 1-8";

pin in s32 currenttoolnumber            "Receives old tool number";

pin out bit toolchanged = false         "Sends signal when tool change finished";

pin out bit delaystart = false          "Starts timerdelay";

pin in bit delaydone =false             "Signals timer finished";

//pin in bit opto1 = false                "State of opto sensor 1";

//pin in bit opto2 = false                "State of opto sensor 2";

//pin in bit opto3 = false                "State of opto sensor 3";

pin out bit forward = false             "Direction signal";

pin out bit run = false                 "Motor command";

//pin in bit ishomedX = false             "Status of X axis homing";

//pin in bit ishomedZ = false             "Status of Z axis homing";

pin out s32 position = 0                "Initialised as a pin for debugging so we can check where it thinks it is";

//param rw float times = 500              "Number of polls of progress_levels 1 & 3 before beginning next move - gives delay for relays";

// Internal and debugging stuff

pin out s32 progress_level = 0;         // tracks the progress of the toolchange, just here so it can be read easily

param rw s32 tnumber = 0;               // Internal toolnumber to allow overrun of quadrant by 1 then reverse back onto it

variable bool bWarn = false;            // first toolnumber reminder

variable float sleeptime = 0;           // our own timer to set delay between progress levels 1 and 2

option singleton yes;                   // makes no sense to have more than one of these components running - only one ATC

function _;

author "smdubovsky@gmail.com";

license "GPL";

;;

/**************************************************************************************

Variables

**************************************************************************************/

bool ls26;	// LS 26 switch state (turret unclamp)

bool ls27;	// LS 27 switch state (turret clamp)

int req_tool_number;	// requested tool number (from LCNC)

enum enumStateMachine {

eStart = 0,		// must be zero

eIdle,

eUnclamp,

eRotateCW,

eRotateCCW,

eClampStart,

eClampFinish

};

enum enumStateMachine state;

enum enumRotateDirection {

edirNone = 0,		// must be zero

edirCW,

edirCCW

};

/**************************************************************************************

Functions

**************************************************************************************/

/**************************************************************************************

need an 'abstract' function that returns the turret clamp state

This makes it much easier to read and you can invert the actual switch state if its reads backwards

*/

bool turret_is_unclamped(void) {

if (ls26 == true) return(true);	// turret unclamped

else return(false);		// turret not unclamped

}

/**************************************************************************************

return the turret clamp complete state

*/

bool turret_is_clamped(void) {

if (ls27 == true) return(true);	// turret clamp complete

else return(false);		// turret clamp not finished

}

/**************************************************************************************

Unclamp the turret

*/

void unclamp_turret(void) {

//enable S201 to unclamp turret

//... (do that here)

}

/**************************************************************************************

clamp the turret

*/

void clamp_turret(void) {

//disable S201 to clamp turret

//... (do that here)

}

/**************************************************************************************

override cam valve and rotate if argument is true

*/

void enable_rotate(bool go) {

if (go) {

// turn on S203

} else {

// turn off S203

}

}

/**************************************************************************************

select rotation direction

*/

void select_rotate_dir(enum enumRotateDirection dir) {

switch(dir){

default:	// so None is also the default state

case edirNone:

//disable S202

//disable S204

break;

case edirCW:

//disable S202

//enable S204

break;

case edirCCW:

//disable S204

//enable S202

break;

}

}

/**************************************************************************************

this function needs to look at the switches and return the tool position we are currently in

*/

int actual_tool_position(void){

//...

return(0);	// return something useful instead of just 0 ;)

}

/**************************************************************************************

this function needs to look at the switches and return the tool position we are currently in

*/

bool index_pos_detected(void){

// look at PS29 switch and return true if index position detected

return(true);	// debug for now

}

/**************************************************************************************

the actual tool changer function that gets called at periodic rate from LCNC

*/

//void puma_tool_changer(void) {

FUNCTION(_)

{

switch(state) {

case eStart:

// this is the startup state.  The turret could be in ANY position and even unlocked.

// Need to get it into a 'known' state.

if (turret_is_clamped()) {

// turret started in fully clamped state.  Set some default valves and go to idle

clamp_turret();

enable_rotate(false);		// stop

select_rotate_dir(edirNone);	// select no direction

motor_freehwheel(false);	// turn off the motor freewheel

state = eIdle;

} else {

state = eUnclamp;

// Note: we could have a special state and leave it unclamped (and stationary) until a tool request is made

// or bump it to first adjacent tool and clamp.  It all depends on what you want it to do if the machine is

// reenabled from an estop and the turret was left in an indeterminate state.  Maybe safer to NOT move it

// until asked to do so.

}

break;

case eIdle:

// this is where you are sitting waiting for a tool change request

// Im not sure what LCNC will pass to us for a tool request.

// (Tool number != current tool) maybe?

if (req_tool_number != actual_tool_position()) state = eUnclamp;	// someone is asking for a different tool position

break;

case eUnclamp:

// unclamp the turret and wait for it to happen before going to next state

unclamp_turret();

if (turret_is_unclamped()) {

state = eRotateCW;	// for now just rotate CW but should decide shortest direction

} else {

// wait until unclamp is sensed

// Note: could keep track of time and if the turret hasn't unclamped in X sec, signal a fault

}

break;

case eRotateCW:

select_rotate_dir(edirCW);	// select CW direction valve

if (req_tool_number == actual_tool_position()) {

enable_rotate(false);		// stop

if (index_pos_detected()) state = eClampStart;	// if index detected, start clamping

} else {

enable_rotate(true);		// spin

}

break;

case eClampStart:

// clamp the turret and wait for it to get close before going to next state

clamp_turret();

select_rotate_dir(edirNone);	// select no direction

if (turret_is_within_3mm()) {

motor_freehwheel(true);	// let the motor freewheel

state = eClampFinish;

} else {

// wait until within 3mm

// Note: could keep track of time and if the turret hasn't unclamped in X sec, signal a fault (or 'bump' the turret)

}

break;

case eClampFinish:

if (turret_is_clamped()) {

motor_freehwheel(false);	// turn off the motor freewheel

state = eIdle;			// go back to idle and wait to do it all over again

} else {

// wait

// Note: could keep track of time and if not complete in X sec, signal a fault (or 'bump' the turret and retry)

}

break;

default:

// default state if the correct stae isn't in this list

// basically this is a big programming error!  You forgot a state!

// should probably generate some error to LCNC

turret_state = eStart;

break;

}

}