function OnAbout(event)

ctrl = event:GetTextCtrl()

ctrl:AppendText("LinuxCNC post processor\n")

ctrl:AppendText("\n")

ctrl:AppendText("Modal G-codes and coordinates\n")

ctrl:AppendText("Comments enclosed with ( and )\n")

ctrl:AppendText("Incremental IJ\n")

ctrl:AppendText("uses G43 tool length offsets\n")

end

--  revision 3/6/09

--  Removed line numbers (they can conflict with 0-word subroutines)

--  revision 3/2/07

--  Removed final safety move. This is now done in SheetCam

--   revision 7/10/04

--   Added new arc handling

--   Created 30/3/2005

--   based on Mach2.post

function OnInit()

post.SetCommentChars ("()", "[]")  --make sure ( and ) characters do not appear in system text

post.Text (" (Filename: ", fileName, ")\n")

post.Text (" (Post processor: ", postName, ")\n")

post.Text (" (Date: ", date, ")\n")

if(scale == metric) then

post.Text (" G21 (Units: Metric)\n") --metric mode

else

post.Text (" G20 (Units: Inches)\n") --inch mode

end

post.Text (" G40 G90\n F1\n")

bigArcs = 1 --stitch arc segments together

minArcSize = 0.05 --arcs smaller than this are converted to moves

end

function OnNewLine()

post.Text ("N")

post.Number (lineNumber, "0000")

lineNumber = lineNumber + 10

end

function OnFinish()

post.Text (" M05 M30\n")

end

function OnRapid()

post.ModalText (" G00")

post.ModalNumber (" X", endX * scale, "0.0000")

post.ModalNumber (" Y", endY * scale, "0.0000")

post.ModalNumber (" Z", (endZ + toolOffset) * scale, "0.0000")

post.Eol()

end

function OnMove()

post.ModalText (" G01")

post.ModalNumber (" X", endX * scale, "0.0000")

post.ModalNumber (" Y", endY * scale, "0.0000")

post.ModalNumber (" Z", (endZ + toolOffset) * scale, "0.0000")

post.ModalNumber (" F", feedRate * scale, "0.###")

post.ModalNumber (" S", spindleSpeed, "0.##")

post.Eol()

end

function OnArc()

if(arcAngle <0) then

post.ModalText (" G03")

else

post.ModalText (" G02")

end

post.NonModalNumber (" X", endX * scale, "0.0000")

post.NonModalNumber (" Y", endY * scale, "0.0000")

post.ModalNumber (" Z", (endZ + toolOffset) * scale, "0.0000")

post.Text (" I")

post.Number ((arcCentreX - currentX) * scale, "0.0000")

post.Text (" J")

post.Number ((arcCentreY - currentY) * scale, "0.0000")

post.ModalNumber (" F", feedRate * scale, "0.0###")

post.ModalNumber (" S", spindleSpeed, "0.##")

post.Eol()

end

function OnSpindleCW()

post.Text (" M03")

post.ModalNumber (" S", spindleSpeed, "0.##")

post.Eol()

end

function OnSpindleCCW()

post.Text (" M04")

post.ModalNumber (" S", spindleSpeed, "0.##")

post.Eol()

end

function OnSpindleOff()

post.Text (" M05\n")

end

function OnNewOperation()

post.Text (" (Operation: ", operationName, ")\n")

if (plungeRate <= 0) then

post.Warning("WARNING: Plunge rate is zero")

end

if (feedRate <= 0) then

post.Warning("WARNING: Feed rate is zero")

end

end

function OnComment()

post.Text(" (",commentText,")\n")

end

function OnToolChange()

post.Text (" M6 T")

post.Number (tool, "0")

post.Text ("  (", toolName, ")\n")

post.Text (" G43 H")

post.Number (tool, "0")

post.Eol()

end

function OnSpindleChanged()

if (spindleSpeed <= 0) then

post.Warning("WARNING: Spindle speed is zero")

end

end

function OnNewPart()

post.Text(" (Part: ",partName,")\n");

end

function OnFloodOn()

post.Text(" M08 (Flood coolant on)\n")

end

function OnMistOn()

post.Text(" M07 (Mist coolant on)\n")

end

function OnCoolantOff()

post.Text(" M09 (Coolant off)\n")

end

function OnDrill()

OnRapid()

depth = drillStart

buffer = plungeSafety

endZ = depth + buffer

OnRapid()

if(drillRetract < buffer) then

buffer = drillRetract

end

while depth > drillZ do

OnRapid()

depth = depth - drillPeck

if (depth < drillZ) then

depth = drillZ

end

endZ = depth

OnMove()

if (depth > drillZ) then --retract if we need to take another bite

endZ = endZ + drillRetract

if (endZ > safeZ) then

endZ = safeZ

end

OnRapid()

end

endZ = depth + buffer

end

if (endZ < safeZ) then

endZ = safeZ

OnRapid()

end

end

function OnRigidTap()

OnRapid()

endZ = drillStart + plungeSafety

OnRapid()

post.ModalText(" G33.1")

post.ModalNumber(" Z", (drillZ + toolOffset) * scale, "0.000")

post.NonModalNumber(" K", tapPitch * scale, "0.0000")

post.Eol()

end